Features:

• PMP technology based on White Moire fringe light，height resolution 0.37um

• Dual 3D Projection systems to avoid shadow and reflection

•Digital High Speed Camera 5M Pix，and 12M, 20M for options

• High Speed inspection capability

• High accuracy and repeatability（CPK 1.67，GRR<1%@3sigma)

• Auto warpage detection

• Solder paste or red gum detection

• 2D/3D true color image display

• Simple Programming, average 5-10 mins

• Gerber 274D/X、CAD，Offline program，Bare Board Teach

• Powerful SPC software

• Automatic 1D 2D Bar Code reading

• Support close loop function with printer machine

• Support MES connection

Specifications

| CATEGORY | ITEM |  | Specifications |
| --- | --- | --- | --- |
| Inspection System | Camera |  | Digital High Speed CCD 5M Pixel (12MP for Option) |
|  | Lamp-house |  | Ring R/G/B/W LED |
|  | FOV |  | 45*36mm(18μm） @5MP 38*30mm(15μm） @5MP 48*36mm(12μm) @12MP 40*30mm(10μm) @12MP |
|  | Speed Per FOV |  | 0.3~0.45s |
|  | Items |  | Paste Solder/Red gum: Volume、Area、Height、Shape、Shift、Bridge、Overflow |
|  | Defects |  | Missing、Shift、Excessive, insufficient, Bridge、Pull、Collapse, Shape、Gold finger、Height、Overflow |
|  | Height Range |  | 1~1000um |
|  | Area Range |  | 0.15mm×0.15mm~10mm×10mm |
|  | Height Detection Resolution |  | 0.37um |
|  | Height Accuracy on Certificated Target |  | 1um |
|  | Height/Area/Volume Repeatability |  | <1%@3sigma |
|  | GR&R |  | <10% |
| Mechanism System | PCB Conveyor |  | Bottom-up fix; Auto transmit and width correct based on SMEMA;Height: 900±20mm |
|  | PCB Size |  | 50x50mm~510x460mm (Larger for option on cost) |
|  | PCB Thickness |  | 0.5~5.0mm |
|  | Max PCB Warp |  | <2mm |
|  | X,Y Platform | Driver | AC servo system |
|  |  | Orientation | <1μm |
|  |  | Moving Speed | 700mm/s |
| Software System | Operation System |  | Microsoft Windows 10 proX64 |
|  | Control& Recognizing | Characteristics | Dual 3D Projection，avoid shadow and reflection |
|  |  | Operation | Graphic programming, Chinese, English |
|  |  | Result | 2D/3D true color |
|  |  | MARK | 2 Common Marks can be chosen, or muti-marks can be chosen and skip function of array boards |
|  |  | Programming | Gerber 274D/X、CAD，Offline program，Bare Board Teach |
|  |  | Barcode Recognition | One dimensional code, 2D Barcode |
|  |  | MES Support | Standard port available |
|  | SPC | Offline SPC | Support |
|  |  | SPC Chart | Any Time Chart(（excel、jpg、bmp ），histogram and control chart to show volume、area、height、shift |
| Control System | Computer |  | Industrial computer：Intel Quad-core CPU Memory:32G，Hard disk:2TB |
|  | Display |  | 19 inch TFT |
| Others | Air Supply |  | 0.5MPA,40cm³/min |
|  | Machine Dimensions(L*W*H) |  | 1148x1260x1660mm |
|  | Weight |  | About 750 KG |
|  | Power Supply |  | AC220V±10%，50/60Hz，1200W |
|  | Working Environment |  | Temperature: 10-40℃，Humidity: 30-80%RH |